DEFIANT  SECURITY  SYSTEMS

~ a division of Longstart Limited ~

www.longstart.co.uk

# THE DEFIANT GROUND ANCHOR

### Fitting Instructions

Tools required:

Hammer Drill

Suitable eye protection – goggles or safety glasses

Standard mastic gun

8mm & 20mm masonry drill bits

Pencil, scissors

1.  Choose the location for your anchor:

Ideally close to a wall to make an attack more difficult.

Fit the anchor into solid concrete or good quality brickwork.

Ensure the fitting will not affect any sub-surface services, e.g. electricity, or gas.

Try to avoid the anchor being a trip hazard if at all possible.

2.  Cut off the template below and lay it exactly where you want the anchor to go.

3.  Put on your eye protection and drill one 8mm diameter hole to a depth of 90mm at point A, making sure the drill is absolutely vertical.  Drop the pencil through the template into the hole to secure the template and drill the second 8mm diameter hole at Point B to a depth of 90mm, again making sure the drill is absolutely vertical.

4.  Remove the pencil and template.  Re-drill the holes to a depth of 90mm using the 20mm diameter drill bit.

5.  Clean the dust and debris out of the holes and check that the anchor pins fit and that the base plate is flat to the floor or wall surface, then using the instructions supplied with the chemical pack and the mastic gun, charge both holes with the chemical bonding agent.  It is essential that the black plastic extender tube is used in the mastic gun to combine both parts of the chemical bonding agent in the clear nozzle.  Properly mixed chemical bonding agent is grey-blue in colour.

6.  Slowly, but firmly, push the ground anchor into place, wiping away any residue when the ground anchor is fully home.  Your ground anchor is now fitted and ready for use after the curing times listed within the chemical instructions.

------------------------------------------------------------------------------------------------------------------------